SeaDrop

Specification & Implementation Document

Version 1.0  —  June 2026

SeaDrop is a portable, pocket-sized file transfer device powered by an ESP32 microcontroller with a TFT touchscreen and microSD slot. It creates its own WiFi access point, allowing an Android phone and a Windows laptop on completely different networks — different routers, different ISPs — to transfer files between each other without either device switching its primary internet connection.

This document covers the full hardware specification, ESP32 firmware architecture, Android application, Windows application, and the SeaDrop Transfer Protocol (STP) that ties all three together.

1. System Overview

1.1 Design Goals

- No manual WiFi switching on either device at any time

- Internet connection on both devices remains uninterrupted during transfer

- One-time device registration via USB. Zero configuration after that

- Transfer initiated from the ESP32 touchscreen, not from either host device

- Works anywhere SeaDrop has power — no router, no infrastructure required

- File transfer via microSD relay: sender writes, receiver reads

1.2 How It Works — User Perspective

1. Plug SeaDrop into any USB power source. It boots and broadcasts its SSID and BLE beacon.

2. Both the Android app and Windows app detect the BLE beacon and silently connect to SeaDrop's WiFi AP in the background. Neither user touches their WiFi settings.

3. The user selects a file on their device and taps Send in the app, or initiates from the SeaDrop touchscreen.

4. The file is written to SeaDrop's microSD card over TCP. Once fully written, the receiving device is notified and pulls the file.

5. SeaDrop's screen shows both device names, file name, size, and a real-time progress bar. The file is deleted from SD after confirmed receipt.

1.3 Architecture Diagram

┌─────────────────────────────────────────────────────────┐

│                        SeaDrop                          │

│  ESP32 APSTA Mode                                        │

│  ┌──────────────┐   ┌──────────────┐  ┌─────────────┐  │

│  │  SoftAP      │   │  microSD      │  │  TFT LCD    │  │

│  │  192.168.4.1 │   │  File Buffer  │  │  ILI9341    │  │

│  └──────┬───────┘   └──────┬────────┘  └─────────────┘  │

│         │                  │                              │

│  ┌──────┴───────┐   ┌──────┴────────┐                   │

│  │  TCP Server  │   │  BLE Advertise │                   │

│  │  Port 4242   │   │  (Discovery)   │                   │

│  └──────────────┘   └───────────────┘                   │

└────────────┬──────────────────────┬───────────────────── ┘

│                      │

┌────────┴───────┐    ┌─────────┴──────────┐

│  Android App   │    │   Windows App       │

│  TCP Client    │    │   TCP Client        │

│  BLE Scanner   │    │   BLE Scanner       │

│  Network Bind  │    │   Route Table Mgmt  │

│  (keeps 4G/WiFi│    │   (keeps own WiFi)  │

│   internet)    │    │                     │

└────────────────┘    └─────────────────────┘

2. Hardware

2.1 Components

| Microcontroller | ESP32-WROOM-32 (XX5R69 module, 38-pin DevKit) |
| --- | --- |
| USB-UART | CP2102 (onboard, used for flashing and USB power) |
| Display | 2.4" TFT LCD ILI9341, 240x320, Arduino Uno shield form factor |
| Touch Controller | XPT2046 (resistive, onboard on shield) |
| Storage | microSD via SPI (on shield) — 16GB card |
| Power Input | USB Micro-B (5V from host or powerbank) |

2.2 Pin Mapping — ESP32 to TFT Shield

The ILI9341 shield uses an 8-bit parallel interface for the display and SPI for the SD card. The shield has onboard level shifters and is 3.3V/5V compliant. Connect 5V to VIN on the ESP32 — never to 3V3.

| Shield Pin | ESP32 GPIO | Notes |
| --- | --- | --- |
| LCD_D0 | GPIO 12 | Parallel data bit 0 — strapping pin, see note below |
| LCD_D1 | GPIO 13 | Parallel data bit 1 |
| LCD_D2 | GPIO 26 | Parallel data bit 2 |
| LCD_D3 | GPIO 25 | Parallel data bit 3 |
| LCD_D4 | GPIO 17 | Parallel data bit 4 |
| LCD_D5 | GPIO 16 | Parallel data bit 5 |
| LCD_D6 | GPIO 27 | Parallel data bit 6 |
| LCD_D7 | GPIO 14 | Parallel data bit 7 |
| LCD_RS | GPIO 2 | Register select (command/data) |
| LCD_WR | GPIO 4 | Write strobe |
| LCD_RD | GPIO 15 | Read strobe |
| LCD_CS | GPIO 33 | Display chip select |
| LCD_RST | GPIO 32 | Display reset |
| SD_SS | GPIO 5 | SD card chip select (SPI) |
| SD_DI | GPIO 23 | SD card MOSI |
| SD_DO | GPIO 19 | SD card MISO |
| SD_SCK | GPIO 18 | SD card SPI clock |
| 5V | VIN | 5V from USB — NOT 3V3 |
| GND | GND | Common ground |

GPIO 12 Note: GPIO 12 is a strapping pin on ESP32-WROOM-32. If HIGH at boot, the ESP32 selects a different flash voltage and will crash. Since LCD_D0 is only driven after boot, this is safe in normal operation. As a precaution, add a 10kΩ pull-down resistor between GPIO 12 and GND.

Forbidden Pins: Never connect anything to GPIO 6, 7, 8, 9, 10, or 11. These are internally wired to the ESP32 flash chip.

3. ESP32 Firmware

3.1 Framework and Libraries

| Framework | Arduino core for ESP32 (espressif/arduino-esp32) |
| --- | --- |
| Display | TFT_eSPI by Bodmer — parallel 8-bit mode |
| Touch | XPT2046_Touchscreen |
| SD | SD.h (Arduino built-in) |
| BLE | NimBLE-Arduino (lighter than built-in BLE stack) |
| WiFi | WiFi.h (Arduino ESP32 core) |
| Storage/NVS | Preferences.h (NVS key-value store) |
| RTOS | FreeRTOS (built into ESP-IDF/Arduino core) |

3.2 TFT_eSPI Configuration

In User_Setup.h inside the TFT_eSPI library folder:

#define ILI9341_DRIVER

#define TFT_PARALLEL_8_BIT

#define TFT_D0  12

#define TFT_D1  13

#define TFT_D2  26

#define TFT_D3  25

#define TFT_D4  17

#define TFT_D5  16

#define TFT_D6  27

#define TFT_D7  14

#define TFT_CS   33

#define TFT_DC    2

#define TFT_RST  32

#define TFT_WR    4

#define TFT_RD   15

3.3 WiFi Configuration — APSTA Mode

The ESP32 runs in WIFI_AP_STA mode. The SoftAP is the transport layer for all device connections. The STA interface is reserved for future OTA updates but does not interfere with AP operation.

WiFi.mode(WIFI_AP_STA);

WiFi.softAP(SEADROP_SSID, SEADROP_PASS);

// SoftAP always on 192.168.4.1

// SSID format: SeaDrop-XXXXXX (last 6 chars of MAC)

The SSID is derived from the ESP32 MAC address at first boot and stored in NVS. It never changes after that, so registered devices can always find it.

3.4 BLE Advertisement

The ESP32 advertises a custom BLE service UUID so Android and Windows apps can detect SeaDrop presence without connecting to WiFi first. BLE advertisement runs continuously and independently of WiFi.

Service UUID:  0xFEAD  (SeaDrop custom)

Advertised data: device name + SSID + connection count (0/1/2)

When a device app scans BLE and finds this UUID, it reads the advertised SSID and connects to SeaDrop's WiFi AP silently in the background.

3.5 TCP Server

SeaDrop runs a TCP server on port 4242 on its AP interface (192.168.4.1:4242). All file transfers use this single endpoint. The server handles two simultaneous clients — one sender, one receiver.

Connection flow:

- Client connects and sends a HELLO packet with device type (ANDROID or WINDOWS) and registered device name

- ESP32 stores the connection in a session table and updates the TFT display

- When both sender and receiver are connected and a transfer is initiated, ESP32 coordinates the handshake via NOTIFY packets to both

- Sender streams file to ESP32 which writes directly to microSD

- On write completion, ESP32 sends READY to receiver

- Receiver pulls file from ESP32 via TCP read

- ESP32 sends COMPLETE to both, deletes file from SD, updates display

3.6 NVS — Persistent Storage

All persistent state lives in ESP32 NVS via Preferences.h:

| seadrop/ssid | Generated SSID (e.g. SeaDrop-A3F9C2) |
| --- | --- |
| seadrop/pass | AP password (set via USB on first flash) |
| seadrop/devA | Registered Android device name |
| seadrop/devW | Registered Windows device name |
| seadrop/tokA | Auth token for Android client |
| seadrop/tokW | Auth token for Windows client |

Tokens are 16-byte random values generated at registration time. Each app stores its own token and sends it in every HELLO packet. ESP32 validates against NVS. Unknown tokens are rejected.

3.7 FreeRTOS Task Structure

| Task: tcp_server_task | Core 0 — TCP server, session management, file relay coordination |
| --- | --- |
| Task: ble_adv_task | Core 0 — BLE advertisement, runs independently |
| Task: display_task | Core 1 — TFT rendering, touch input polling, UI state machine |
| Task: sd_write_task | Core 1 — microSD write/read, triggered by tcp_server_task via queue |

Inter-task communication uses FreeRTOS queues. The TCP task pushes file metadata and chunk pointers to the SD task queue. The SD task signals completion back via a semaphore. This prevents blocking the TCP receive buffer during slow SD writes.

3.8 Display UI — State Machine

| STATE_IDLE | Shows SeaDrop logo, SSID, connected device count (0/1/2) |
| --- | --- |
| STATE_CONNECTED | Shows both device names once both apps are connected |
| STATE_CONFIRM | Transfer initiated — shows filename, size, sender/receiver, Confirm button |
| STATE_TRANSFER | Progress bar, bytes transferred / total, transfer speed in KB/s |
| STATE_DONE | Shows DONE with filename and time taken. Returns to IDLE after 3 seconds |
| STATE_ERROR | Shows error type (SD_FULL, TIMEOUT, REJECTED). Returns to IDLE after 5 seconds |

3.9 USB Registration Flow

On first connection via USB Serial (115200 baud), the firmware enters REGISTRATION mode if NVS tokens are empty. The companion app or a serial terminal sends:

REG ANDROID <device_name>

REG WINDOWS <device_name>

Firmware generates a random token, stores it in NVS, and responds:

TOKEN <16_byte_hex>

The app stores this token permanently. Registration only needs to happen once per device. Subsequent connections use the stored token for authentication.

4. SeaDrop Transfer Protocol (STP)

STP is a minimal custom TCP protocol over port 4242. All packets are newline-terminated ASCII command lines followed by binary payloads where applicable. This keeps the protocol debuggable with a terminal and avoids parser complexity on the ESP32.

4.1 Packet Format

COMMAND [PARAM1] [PARAM2] ... \n

[binary payload if applicable]

Commands are uppercase ASCII. Parameters are space-separated. Binary payloads follow immediately after the newline with length specified in the command.

4.2 Command Reference

| HELLO <type> <name> <token> | Client greeting. type = ANDROID or WINDOWS. Sent on connect. |
| --- | --- |
| HELLO_ACK <session_id> | ESP32 response. Assigns session ID for this connection. |
| SEND <filename> <size_bytes> <crc32> | Client declares intent to send a file. |
| SEND_ACK | ESP32 ready to receive. Client begins streaming bytes. |
| SEND_DONE | Client signals end of file stream. |
| NOTIFY <filename> <size_bytes> | ESP32 to receiver: file is ready to pull. |
| PULL <filename> | Receiver requests file from ESP32. |
| PULL_DATA <size_bytes> | ESP32 begins streaming file bytes to receiver. |
| PULL_DONE | ESP32 signals end of file stream. |
| ACK | Generic acknowledgement. |
| REJECT <reason> | Refusal with reason code: AUTH_FAIL, SD_FULL, NO_PEER, BUSY. |
| CANCEL | Either party cancels an in-progress transfer. |
| STATUS | Client requests current session state. |
| STATUS_RESP <state> <peer_name> | ESP32 response to STATUS query. |
| PING | Keepalive from client. |
| PONG | ESP32 response to PING. |

4.3 Transfer Sequence — Full Flow

Android                    ESP32                      Windows

|                         |                           |

|--- HELLO ANDROID ... -->|                           |

|<-- HELLO_ACK sid_1 -----|                           |

|                         |<-- HELLO WINDOWS ... -----|

|                         |--- HELLO_ACK sid_2 ------>|

|                         |                           |

|--- SEND file.mp4 ... -->|                           |

|<-- SEND_ACK ------------|                           |

|--- [binary stream] ---->|                           |

|--- SEND_DONE ---------->|--- NOTIFY file.mp4 ------>|

|                         |<-- PULL file.mp4 ----------|

|                         |--- PULL_DATA size_bytes -->|

|                         |--- [binary stream] ------->|

|                         |--- PULL_DONE ------------->|

|<-- ACK -----------------|<-- ACK --------------------|

|                         | (SD file deleted)          |

4.4 CRC32 Integrity Check

The sender computes CRC32 of the entire file and includes it in the SEND command. After writing to SD, ESP32 recomputes CRC32 and rejects if mismatch. Receiver similarly validates after PULL_DONE. If either check fails, ESP32 sends REJECT CHECKSUM_FAIL and the transfer restarts from the beginning.

5. Android Application

5.1 Tech Stack

| Language | Kotlin |
| --- | --- |
| Min SDK | API 26 (Android 8.0) |
| Target SDK | API 34 (Android 14) |
| BLE | Android BluetoothLeScanner API |
| WiFi | WifiManager — connectToNetwork() with NetworkSpecifier |
| Network Routing | ConnectivityManager.bindProcessToNetwork() |
| Background | Foreground Service (persistent notification) |
| File Access | Storage Access Framework (SAF) — no root required |
| TCP | Java Socket / Kotlin coroutines |
| Notification | NotificationCompat — transfer progress |

5.2 Required Permissions

ACCESS_FINE_LOCATION

CHANGE_WIFI_STATE

ACCESS_WIFI_STATE

BLUETOOTH_SCAN          (API 31+)

BLUETOOTH_CONNECT       (API 31+)

NEARBY_WIFI_DEVICES     (API 33+)

FOREGROUND_SERVICE

INTERNET

READ_MEDIA_IMAGES / READ_MEDIA_VIDEO / READ_MEDIA_AUDIO  (API 33+)

5.3 The Critical Network Binding Call

This single API call is what makes SeaDrop transfers work without dropping Android internet. Without it, Android routes all traffic through whichever network it considers 'best' — and since SeaDrop's AP has no internet, Android would disconnect from it automatically.

val network = connectivityManager.getNetworkForRequest(seadropRequest)

connectivityManager.bindProcessToNetwork(network)

This binds only the SeaDrop transfer socket to the SeaDrop network. All other app traffic and system internet traffic continues through the phone's original connection unaffected. After transfer completes, bindProcessToNetwork(null) releases the binding.

This API requires API 23+. On API 26 (our minimum) it is fully reliable.

5.4 Service Architecture

The app runs a persistent Foreground Service called SeaDropService. It starts on boot if a registered SeaDrop token exists in SharedPreferences. The service handles:

- BLE scan loop — polls for SeaDrop advertisement UUID 0xFEAD every 10 seconds when not connected

- Silent WiFi connection to SeaDrop SSID when beacon detected

- TCP connection to 192.168.4.1:4242 and HELLO handshake

- Incoming NOTIFY handling — shows system notification asking user to accept or reject incoming file

- Outgoing SEND — initiated from share sheet or in-app file picker

- Transfer progress updates to notification and optionally to in-app UI

5.5 User Flow

Sending: user selects file via share sheet (Send via SeaDrop) or in-app picker. If SeaDrop is detected, transfer begins immediately. If not detected, app shows 'Searching for SeaDrop...' and initiates when found within 30 seconds.

Receiving: incoming file triggers a system notification 'File incoming from [device name]: [filename] ([size])' with Accept and Reject buttons. Accept triggers PULL sequence. File saved to Downloads/SeaDrop/ by default, configurable.

6. Windows Application

6.1 Tech Stack

| Language | C# (.NET 8) |
| --- | --- |
| UI Framework | WinUI 3 (Windows App SDK) — system tray + minimal window |
| BLE | Windows.Devices.Bluetooth.Advertisement namespace |
| WiFi | Windows.Devices.WiFiDirect OR Native WiFi API (wlanapi.dll) |
| Route Management | CreateIpForwardEntry2 (Win32 IP Helper API) |
| TCP | System.Net.Sockets |
| Background | Windows Service OR startup task via Task Scheduler |
| Installer | MSIX package — handles elevation and startup registration |
| Min OS | Windows 10 19041 (20H1) — WiFi Direct API stable from this build |

6.2 WiFi Connection Without Interrupting Internet

Windows creates a Microsoft Wi-Fi Direct Virtual Adapter as a separate virtual network interface when connecting to a WiFi Direct or AP network via the WiFiDirect API. This is separate from the physical WiFi adapter.

After connecting to SeaDrop's AP via this virtual adapter, the Windows app calls the IP Helper API to add a host route:

Destination: 192.168.4.0/24

Gateway:     SeaDrop virtual adapter

Metric:      1  (high priority for this specific subnet only)

This tells Windows: traffic for 192.168.4.x goes through SeaDrop. All other traffic — including internet — continues on the original WiFi adapter with its original default gateway. The user notices nothing. After transfer, the app removes this route entry.

The MSIX installer requests elevated privileges once at install time. The app itself runs without elevation after that, using a pre-approved capability declared in the app manifest.

6.3 BLE Discovery

Windows 10/11 supports BLE advertisement watching natively via Windows.Devices.Bluetooth.Advertisement.BluetoothLEAdvertisementWatcher. The app watches for the SeaDrop service UUID 0xFEAD in the background.

var watcher = new BluetoothLEAdvertisementWatcher();

watcher.AdvertisementFilter.Advertisement.ServiceUuids.Add(

Guid.Parse("0000FEAD-0000-1000-8000-00805F9B34FB"));

watcher.Received += OnSeaDropDetected;

watcher.Start();

6.4 User Flow

The app lives in the system tray. Icon shows SeaDrop connection state: grey (not found), yellow (searching), green (connected and ready).

Sending: right-click tray icon, Send File, file picker, done. Or drag a file onto the tray icon. Transfer begins immediately if SeaDrop is connected.

Receiving: Windows toast notification appears: 'Incoming file from [device]: [filename] ([size])' with Accept/Reject. Accepted files save to Downloads\SeaDrop\ by default.

Context menu entry 'Send via SeaDrop' is registered in the Windows shell for right-click on any file in Explorer. This is registered by the MSIX installer.

7. One-Time Registration Flow

Registration happens once per device over USB serial. It writes auth tokens to both the ESP32 NVS and the host device's app storage. After registration, all communication is wireless and automatic.

1. Connect SeaDrop to PC via USB cable

2. Open SeaDrop app on Windows (or Android via USB-OTG if supported)

3. App detects SeaDrop on COM port, opens serial at 115200 baud

4. App sends:  REG WINDOWS MyLaptop

5. ESP32 responds:  TOKEN 4a7f2c8e1b9d3f5a  (16-byte hex)

6. App stores token in local secure storage

7. Repeat for Android: REG ANDROID MyPhone

8. Registration complete — USB can be disconnected

The SeaDrop TFT display shows 'Registration Mode' during this process and 'Registration Complete' when both devices are registered. If a device is already registered, re-registering overwrites its token — useful for replacing a device.

8. Security Model

SeaDrop is not designed as a high-security system. It is designed to be as secure as AirDrop — which is to say, proximity-based trust with token authentication.

| Auth tokens | 16-byte random tokens stored in NVS. Required on every TCP connection. Unknown tokens are dropped immediately. |
| --- | --- |
| AP password | WPA2 password set during firmware flash. Prevents random devices connecting to SeaDrop AP. |
| Physical proximity | BLE has ~10 meter range. WiFi AP is intentionally low power. Transfers are inherently proximity-gated. |
| Transfer consent | Receiver must explicitly accept every incoming file. ESP32 touchscreen shows file details and requires physical confirmation for large files (>50MB). |
| No cloud, no relay | SeaDrop has no internet connectivity. There is no server. Files never leave the local SeaDrop-AP network. |

9. Build Order — 16 Days

This is a solo build. The order below minimizes risk by proving the core data path before building any UI or application layer.

| Days 1–2 | ESP32 firmware: APSTA mode, SoftAP up, TCP server on 4242, hello/ack handshake only. Verify two TCP clients can connect simultaneously while AP is live. |
| --- | --- |
| Days 3–4 | SD card integration. Write a file from TCP stream to SD, read it back out to a second TCP client. Verify no data corruption with CRC32. This is the core relay. |
| Days 5–6 | Android app: BLE scan, silent WiFi connect to ESP32 AP, network binding, TCP HELLO, SEND a file. No UI yet — just logcat confirmation. |
| Days 7–8 | Windows app: BLE watcher, WiFi connect to ESP32 AP, route table entry, TCP HELLO, PULL a file that Android already sent. Verify internet stays live on both. |
| Days 9–10 | Full STP implementation on all three: SEND, NOTIFY, PULL, ACK, REJECT, CANCEL, CRC validation. Bidirectional transfer working. |
| Days 11–12 | ESP32 TFT UI: state machine, device name display, progress bar, confirm button. NVS registration flow via USB serial. |
| Days 13–14 | Android and Windows UI polish: system notifications, tray icon, share sheet integration, file picker, accept/reject flow. |
| Day 15 | End-to-end demo rehearsal. Two devices on different networks. SeaDrop powered by powerbank. Transfer in both directions. Confirm internet uninterrupted on both. |
| Day 16 | Buffer. Something will break. |

10. Known Constraints and Limitations

| Transfer speed | Real-world throughput via ESP32 SoftAP relay: 3–8 Mbps. A 100MB file takes ~15–30 seconds. Acceptable for documents, photos, short video. Large video files (1GB+) will be slow. |
| --- | --- |
| Concurrent transfers | SeaDrop handles one transfer at a time. A second transfer request while one is in progress receives REJECT BUSY. |
| SD card | microSD via SPI shares the SPI bus. SD write speed is the bottleneck at ~2–4 MB/s. SDMMC 4-bit mode would be faster but requires a different wiring harness. |
| Android below API 26 | bindProcessToNetwork behavior is unreliable. Minimum supported is Android 8.0 (API 26). |
| Windows elevation | Route table management requires administrator privileges. MSIX installer handles this at install time. Per-transfer elevation prompts are not acceptable UX. |
| Single AP channel | ESP32 APSTA mode locks both AP and STA to the same channel. If STA is connected to a 2.4GHz router on channel 6, the AP is also on channel 6. This is only relevant if the STA interface is used. |
| File size limit | Limited by microSD free space. With 16GB card and typical use, no practical limit. Large files buffer to SD incrementally — no RAM limit on file size. |

SeaDrop — Specification v1.0 — June 2026